WEILER E-90 × 3000 — Technical Specification Sheet

Model Year: 2012 (platform unchanged 2009–2015)

Spindle Bore: 10.3” (262 mm)

Machine Type: Heavy-Duty Precision Teach-In Lathe

Manufacturer: WEILER Werkzeugmaschinen GmbH, Germany

---

1. MACHINE CAPACITY

- Turning Length: 3000 mm (118.11”)

- Swing Over Bed: 900 mm (35.43”)

- Swing Over Cross Slide: 590 mm (23.23”)

- Bed Width: 600 mm (23.62”)

- Carriage Travel: Full length of bed

---

2. SPINDLE SYSTEM

- Spindle Bore: 262 mm (10.3”)

- Spindle Nose: DIN ISO 702-1, Size 20

- Front Bearing Diameter: 448 mm (17.64”)

- Speed Range:

  - Gear I: 1–180 RPM

  - Gear II: 4–700 RPM

- Main Drive Power:

  - 45 kW (60% duty)

  - 37 kW (100% duty)

- Maximum Spindle Torque: 8,000 Nm

- Spindle Type: Heavy-duty, large-bore oilfield-class

---

3. FEEDS & THREADING

- Feed Range: 0.001–50 mm/rev

- Rapid Traverse:

  - Longitudinal: 10 m/min

  - Transverse: 7 m/min

- Threading Capability:

  - Metric: 0.1–2000 mm

  - Inch: 112–1/64 TPI

  - Modular: 0.05–56 mm

  - Diametral: 448–0.05 DP

- Max Thread Courses: 360

---

4. TAILSTOCK

- Quill Diameter: 140 mm (5.51”)

- Quill Travel: 300 mm (11.81”)

- Quill Taper: MT6

- Tailstock Body: Heavy-duty, fully adjustable

---

5. DIMENSIONS & WEIGHT

- Approx. Machine Dimensions:

  - 5700 × 2600 × 2300 mm

  - (224.41” × 102.36” × 90.55”)

- Machine Weight: ~11,000 kg (24,250 lbs)

(varies with chuck, steady rests, and options)

---

6. ELECTRICAL & CONTROL

- Control Type: WEILER Teach-In / Cycle Control

- Power Requirements: 400–480V, 3-phase

- Drive System: AC vector drive with gearbox

- Safety: CE-compliant guarding, interlocks, e-stop

---

7. STANDARD FEATURES

- Hardened and ground bedways

- High-torque gearbox for large-diameter work

- Automatic lubrication system

- Coolant system

- Heavy-duty carriage and cross slide

- Full universal threading package