WEILER E-70 3000 (8” Spindle Bore) — Technical Specification Sheet

Model Year: Approx. 2012

Machine Type: Precision Engine Lathe

Manufacturer: WEILER Werkzeugmaschinen GmbH

Configuration: Long-bed (3000 mm) with large-bore spindle (≈ 8.5”)

---

1. GENERAL SPECIFICATIONS

- Center Distance: 3000 mm

- Swing Over Bed: 720 mm (28.3”)

- Swing Over Cross Slide: 430 mm (16.9”)

- Bed Width: 480 mm

- Cross Slide Travel: 460 mm

- Tool Size: 32 × 25 mm

---

2. SPINDLE

- Spindle Bore: 216 mm (≈ 8.5”)

- Spindle Nose: DIN ISO 702-1, Size 15

- Front Bearing Diameter: 280 mm

- Internal Taper: 220 metric

- Speed Ranges:

  - Low Range: 1–350 RPM

  - High Range: 4–1200 RPM

- Main Drive Power: 37 kW (60% duty) / 30 kW (100% duty)

- Max Torque: 3,400 Nm

---

3. FEEDS & THREADING

- Feed Range: 0.001–50 mm/rev

- Rapid Traverse:

  - Longitudinal: 10 m/min

  - Transverse: 5 m/min

- Threading Capability:

  - Metric: 0.1–2000 mm

  - Inch: 112–1/64 TPI

  - Modular: 0.05–56 mm

  - Diametral: 448–0.05 DP

- Max Thread Courses: 360

- Feed Force:

  - Longitudinal: 25,000 N

  - Transverse: 13,000 N

---

4. TAILSTOCK

- Quill Diameter: 115 mm

- Quill Travel: 225 mm

- Quill Taper: MT6

---

5. DIMENSIONS & WEIGHT

- Overall Length: Center distance + 2800 mm

- Width: 2760 mm

- Height: 2050 mm

- Weight: 7,400–7,800 kg (varies by configuration)